PROGRAM:  Microwave Low Pass Filter Analysis

clear;

clc;

close all;

%% Parameters

c = 3e8;                  % Speed of light (m/s)

er = 5.6;                 % Dielectric constant

h = 2.9e-3;               % Substrate height (m)

tan_delta = 0.01;         % Loss tangent

fc = 2e9;                 % Cutoff frequency (Hz)

N = 4;                    % Filter order

% Frequency points

f_values = [1 1.5 2 2.5 3 3.5 4]*1e9;

%% Effective dielectric constant

ereff = (er + 1)/2 + ...

(er - 1)/2 * (1 + 12*h)^(-0.5);

% Phase velocity

v = c/sqrt(ereff);

%% Initialize arrays

S11_theory = zeros(size(f_values));

S21_theory = zeros(size(f_values));

S11_practical = zeros(size(f_values));

S21_practical = zeros(size(f_values));

RL_theory = zeros(size(f_values));

RL_practical = zeros(size(f_values));

IL_theory = zeros(size(f_values));

IL_practical = zeros(size(f_values));

VSWR_theory = zeros(size(f_values));

VSWR_practical = zeros(size(f_values));

%% Filter Analysis

for k = 1:length(f_values)

f = f_values(k);

%% Normalized frequency

x = f/fc;

%% Theoretical S21

S21_theory(k) = ...

1/sqrt(1 + x^(2*N));

%% Theoretical reflection coefficient

Gamma_theory = ...

sqrt(1 - S21_theory(k)^2);

S11_theory(k) = Gamma_theory;

%% Practical attenuation

omega = 2*pi*f;

alpha = ...

tan_delta*sqrt(er)*omega/(2*v);

loss = exp(-alpha*h);

%% Practical S21

S21_practical(k) = ...

S21_theory(k)*loss;

%% Practical reflection coefficient

Gamma_practical = ...

sqrt(1 - S21_practical(k)^2);

S11_practical(k) = Gamma_practical;

%% Return Loss

RL_theory(k) = ...

-20*log10(abs(S11_theory(k)));

RL_practical(k) = ...

-20*log10(abs(S11_practical(k)));

%% Insertion Loss

IL_theory(k) = ...

-20*log10(abs(S21_theory(k)));

IL_practical(k) = ...

-20*log10(abs(S21_practical(k)));

%% VSWR

VSWR_theory(k) = ...

(1 + abs(S11_theory(k))) / ...

(1 - abs(S11_theory(k)));

VSWR_practical(k) = ...

(1 + abs(S11_practical(k))) / ...

(1 - abs(S11_practical(k)));

end

%% Display Results

fprintf('\nMICROWAVE LOW PASS FILTER ANALYSIS\n');

fprintf('============================================\n');

fprintf('\nCutoff Frequency = %.2f GHz\n',fc/1e9);

fprintf('Filter Order     = %d\n',N);

fprintf('Dielectric Constant = %.2f\n',er);

fprintf('\n');

fprintf(['Freq(GHz)\tS11(Theory)\tS11(Practical)' ...

'\tS21(Theory)\tS21(Practical)\n']);

fprintf('-------------------------------------------------------------\n');

for k = 1:length(f_values)

fprintf('%.2f\t\t%.4f\t\t%.4f\t\t%.4f\t\t%.4f\n', ...

f_values(k)/1e9, ...

S11_theory(k), ...

S11_practical(k), ...

S21_theory(k), ...

S21_practical(k));

end

%% Return Loss and Insertion Loss Table

fprintf('\n\nRETURN LOSS AND INSERTION LOSS\n');

fprintf('============================================\n');

fprintf(['Freq(GHz)\tRL Theory\tRL Practical' ...

'\tIL Theory\tIL Practical\n']);

fprintf('-------------------------------------------------------------\n');

for k = 1:length(f_values)

fprintf('%.2f\t\t%.2f\t\t%.2f\t\t%.2f\t\t%.2f\n', ...

f_values(k)/1e9, ...

RL_theory(k), ...

RL_practical(k), ...

IL_theory(k), ...

IL_practical(k));

end

%% VSWR Table

fprintf('\n\nVSWR ANALYSIS\n');

fprintf('============================================\n');

fprintf('Freq(GHz)\tVSWR Theory\tVSWR Practical\n');

fprintf('--------------------------------------------\n');

for k = 1:length(f_values)

fprintf('%.2f\t\t%.3f\t\t%.3f\n', ...

f_values(k)/1e9, ...

VSWR_theory(k), ...

VSWR_practical(k));

end

%% =========================================================

% ALL PLOTS IN ONE FIGURE - 2 x 3 SUBPLOT

% =========================================================

figure('Name','Microwave Low Pass Filter Analysis');

%% 1. S21 Response

subplot(2,3,1);

plot(f_values/1e9, S21_theory, ...

'o-b','LineWidth',2);

hold on;

plot(f_values/1e9, S21_practical, ...

's-r','LineWidth',2);

grid on;

xlabel('Frequency (GHz)');

ylabel('|S_{21}|');

title('S_{21} Response');

legend('Theoretical','Practical', ...

'Location','best');

%% 2. S11 Response

subplot(2,3,2);

plot(f_values/1e9, S11_theory, ...

'o-b','LineWidth',2);

hold on;

plot(f_values/1e9, S11_practical, ...

's-r','LineWidth',2);

grid on;

xlabel('Frequency (GHz)');

ylabel('|S_{11}|');

title('S_{11} Response');

legend('Theoretical','Practical', ...

'Location','best');

%% 3. Return Loss

subplot(2,3,3);

plot(f_values/1e9, RL_theory, ...

'o-b','LineWidth',2);

hold on;

plot(f_values/1e9, RL_practical, ...

's-r','LineWidth',2);

grid on;

xlabel('Frequency (GHz)');

ylabel('Return Loss (dB)');

title('Return Loss');

legend('Theoretical','Practical', ...

'Location','best');

%% 4. Insertion Loss

subplot(2,3,4);

plot(f_values/1e9, IL_theory, ...

'o-b','LineWidth',2);

hold on;

plot(f_values/1e9, IL_practical, ...

's-r','LineWidth',2);

grid on;

xlabel('Frequency (GHz)');

ylabel('Insertion Loss (dB)');

title('Insertion Loss');

legend('Theoretical','Practical', ...

'Location','best');

%% 5. VSWR

subplot(2,3,5);

plot(f_values/1e9, VSWR_theory, ...

'o-b','LineWidth',2);

hold on;

plot(f_values/1e9, VSWR_practical, ...

's-r','LineWidth',2);

grid on;

xlabel('Frequency (GHz)');

ylabel('VSWR');

title('VSWR');

legend('Theoretical','Practical', ...

'Location','best');

%% 6. Overall Filter Response

subplot(2,3,6);

plot(f_values/1e9, S21_theory, ...

'o-b','LineWidth',2);

hold on;

plot(f_values/1e9, S21_practical, ...

's-r','LineWidth',2);

xline(fc/1e9,'--k','Cutoff Frequency', ...

'LineWidth',1.5);

grid on;

xlabel('Frequency (GHz)');

ylabel('|S_{21}|');

title('Overall Low Pass Filter Response');

legend('Theoretical','Practical','Cutoff', ...

'Location','best');

%% Overall Figure Title

sgtitle('Microwave Low Pass Filter Analysis');

PROGRAM: HIGH-PASS MICROWAVE FILTER ANALYSIS

clear;

clc;

close all;

%% PARAMETERS

c = 3e8;                  % Speed of light (m/s)

er = 5.6;                 % Dielectric constant - FR4

h = 2.9e-3;               % Substrate height (m)

tan_delta = 0.01;         % Loss tangent

fc = 2e9;                 % Cutoff frequency = 2 GHz

N = 4;                    % Filter order

% Frequency points

f_values = [0.5 1 1.5 2 2.5 3 3.5 4] * 1e9;

%% EFFECTIVE DIELECTRIC CONSTANT

ereff = (er + 1)/2 + ...

(er - 1)/2 * (1 + 12*h)^(-0.5);

% Phase velocity

v = c / sqrt(ereff);

%% INITIALIZE ARRAYS

S11_theory = zeros(size(f_values));

S21_theory = zeros(size(f_values));

S11_practical = zeros(size(f_values));

S21_practical = zeros(size(f_values));

RL_theory = zeros(size(f_values));

RL_practical = zeros(size(f_values));

IL_theory = zeros(size(f_values));

IL_practical = zeros(size(f_values));

VSWR_theory = zeros(size(f_values));

VSWR_practical = zeros(size(f_values));

%% HIGH-PASS FILTER ANALYSIS

for k = 1:length(f_values)

f = f_values(k);

% Normalized frequency

x = fc / f;

% Theoretical S21

S21_theory(k) = ...

1 / sqrt(1 + x^(2*N));

% Theoretical reflection coefficient

Gamma_theory = ...

sqrt(1 - S21_theory(k)^2);

S11_theory(k) = Gamma_theory;

%% PRACTICAL DIELECTRIC LOSS

omega = 2*pi*f;

alpha = ...

tan_delta * sqrt(er) * omega / (2*v);

loss = exp(-alpha*h);

% Practical S21

S21_practical(k) = ...

S21_theory(k) * loss;

% Practical reflection coefficient

Gamma_practical = ...

sqrt(1 - S21_practical(k)^2);

S11_practical(k) = Gamma_practical;

%% RETURN LOSS

RL_theory(k) = ...

-20 * log10(abs(S11_theory(k)));

RL_practical(k) = ...

-20 * log10(abs(S11_practical(k)));

%% INSERTION LOSS

IL_theory(k) = ...

-20 * log10(abs(S21_theory(k)));

IL_practical(k) = ...

-20 * log10(abs(S21_practical(k)));

%% VSWR

VSWR_theory(k) = ...

(1 + abs(S11_theory(k))) / ...

(1 - abs(S11_theory(k)));

VSWR_practical(k) = ...

(1 + abs(S11_practical(k))) / ...

(1 - abs(S11_practical(k)));

end

%% DISPLAY FILTER PARAMETERS

fprintf('\n');

fprintf('===============================================\n');

fprintf('       HIGH-PASS MICROWAVE FILTER ANALYSIS\n');

fprintf('===============================================\n');

fprintf('Cutoff Frequency    = %.2f GHz\n', fc/1e9);

fprintf('Filter Order        = %d\n', N);

fprintf('Dielectric Constant = %.2f\n', er);

fprintf('Substrate Height    = %.2f mm\n', h*1000);

fprintf('Loss Tangent       = %.4f\n', tan_delta);

fprintf('===============================================\n');

%% S-PARAMETER TABLE

fprintf('\n');

fprintf('S-PARAMETER ANALYSIS\n');

fprintf('---------------------------------------------------------------\n');

fprintf('Freq(GHz)\tS11 Theory\tS11 Practical\tS21 Theory\tS21 Practical\n');

fprintf('---------------------------------------------------------------\n');

for k = 1:length(f_values)

fprintf('%.2f\t\t%.4f\t\t%.4f\t\t%.4f\t\t%.4f\n', ...

f_values(k)/1e9, ...

S11_theory(k), ...

S11_practical(k), ...

S21_theory(k), ...

S21_practical(k));

end

%% RETURN LOSS AND INSERTION LOSS TABLE

fprintf('\n');

fprintf('RETURN LOSS AND INSERTION LOSS\n');

fprintf('---------------------------------------------------------------\n');

fprintf('Freq(GHz)\tRL Theory\tRL Practical\tIL Theory\tIL Practical\n');

fprintf('---------------------------------------------------------------\n');

for k = 1:length(f_values)

fprintf('%.2f\t\t%.2f\t\t%.2f\t\t%.2f\t\t%.2f\n', ...

f_values(k)/1e9, ...

RL_theory(k), ...

RL_practical(k), ...

IL_theory(k), ...

IL_practical(k));

end

%% VSWR TABLE

fprintf('\n');

fprintf('VSWR ANALYSIS\n');

fprintf('---------------------------------------------------------------\n');

fprintf('Freq(GHz)\tVSWR Theory\tVSWR Practical\n');

fprintf('---------------------------------------------------------------\n');

for k = 1:length(f_values)

fprintf('%.2f\t\t%.3f\t\t%.3f\n', ...

f_values(k)/1e9, ...

VSWR_theory(k), ...

VSWR_practical(k));

end

%% ALL PLOTS

% 2 x 3 subplot arrangement

figure('Name','High-Pass Microwave Filter Analysis');

%% 1. S21 RESPONSE

subplot(2,3,1);

plot(f_values/1e9, S21_theory, ...

'o-b','LineWidth',2);

hold on;

plot(f_values/1e9, S21_practical, ...

's-r','LineWidth',2);

grid on;

xlabel('Frequency (GHz)');

ylabel('|S_{21}|');

title('S_{21} Response');

legend('Theoretical','Practical', ...

'Location','best');

%% 2. S11 RESPONSE

subplot(2,3,2);

plot(f_values/1e9, S11_theory, ...

'o-b','LineWidth',2);

hold on;

plot(f_values/1e9, S11_practical, ...

's-r','LineWidth',2);

grid on;

xlabel('Frequency (GHz)');

ylabel('|S_{11}|');

title('S_{11} Response');

legend('Theoretical','Practical', ...

'Location','best');

%% 3. RETURN LOSS

subplot(2,3,3);

plot(f_values/1e9, RL_theory, ...

'o-b','LineWidth',2);

hold on;

plot(f_values/1e9, RL_practical, ...

's-r','LineWidth',2);

grid on;

xlabel('Frequency (GHz)');

ylabel('Return Loss (dB)');

title('Return Loss');

legend('Theoretical','Practical', ...

'Location','best');

%% 4. INSERTION LOSS

subplot(2,3,4);

plot(f_values/1e9, IL_theory, ...

'o-b','LineWidth',2);

hold on;

plot(f_values/1e9, IL_practical, ...

's-r','LineWidth',2);

grid on;

xlabel('Frequency (GHz)');

ylabel('Insertion Loss (dB)');

title('Insertion Loss');

legend('Theoretical','Practical', ...

'Location','best');

%% 5. VSWR

subplot(2,3,5);

plot(f_values/1e9, VSWR_theory, ...

'o-b','LineWidth',2);

hold on;

plot(f_values/1e9, VSWR_practical, ...

's-r','LineWidth',2);

grid on;

xlabel('Frequency (GHz)');

ylabel('VSWR');

title('VSWR');

legend('Theoretical','Practical', ...

'Location','best');

%% 6. FILTER CHARACTERISTIC

subplot(2,3,6);

plot(f_values/1e9, IL_theory, ...

'o-b','LineWidth',2);

hold on;

plot(f_values/1e9, IL_practical, ...

's-r','LineWidth',2);

xline(fc/1e9,'--k','Cutoff Frequency', ...

'LineWidth',1.5);

grid on;

xlabel('Frequency (GHz)');

ylabel('Insertion Loss (dB)');

title('High-Pass Filter Characteristic');

legend('Theoretical','Practical','Location','best');

sgtitle('High-Pass Microwave Filter Analysis');